UNIVERSITY OF TABRIZ · FACULTY OF CIVIL ENGINEERING	Preliminary PhD Research Discussion Package

DOCUMENT 01 · PAIRED RESEARCH PROGRAMME SUMMARY

Dynamics of Flow-Excited Hydraulic Gates: Loading and Identification

دینامیک دریچه‌های هیدرولیکی تحت تحریک جریان: بارگذاری و شناسایی

A proposed pair of independent but complementary doctoral concepts

Candidates: Zina · Jamal     Proposed supervisor: Prof. Reza Tarinejad     October 2026

Status Proposed research direction for preliminary discussion, subject to supervisor confirmation. Two points remain open: (1) a short recent-literature confirmation review of both research gaps; (2) confirmation that neither scope overlaps with current students' work.

# 1.  Why this programme

Prof. Tarinejad's publication record contains two active lines that are not yet connected:

- Output-only identification of dam dynamics, most recently transmissibility-based methods [1, 2, 4] and automated SSI [3].

- Hydraulic control structures: gate and weir discharge performance [5, 6] and outlet CFD verification [7].

The hydraulic work characterises time-averaged performance; the identification work addresses dam bodies under seismic or ambient input. Flow-excited gates sit between the two: they are excited by fluctuating flow, and their dynamic properties change with water level and operation.

# 2.  Scientific logic

| FLOW | → | HYDRODYNAMIC LOADING | → | STRUCTURAL DYNAMIC RESPONSE |
| --- | --- | --- | --- | --- |
| Operating state: head, opening, submergence, sill geometry |  | Jamal: what fluctuating loads does the flow apply, by which mechanism, and how do they scale? |  | Zina: how can the gate's modal properties be identified when water and operating state change them? |
| Common hydraulic context |  | Rigid gate · pressures · LES → regime map, scaling, criterion |  | Flexible gate · accelerations · EMA → parameter-varying identification |

Each thesis addresses one link of the chain: Jamal characterises the excitation, Zina identifies the structure. Neither thesis needs the other's results. Zina uses synthetic loads defined independently, and Jamal uses a rigid gate whose dynamics play no role in his question.

# 3.  The two doctoral concepts

|  | ZINA | JAMAL |
| --- | --- | --- |
| Discussion title | Operational Modal Identification of Hydraulic Gates under Varying Flow Conditions | Unsteady Hydrodynamic Loading of Sluice Gates with Bottom Sills |
| Research gap | Transmissibility OMA assumes the structure is unchanged across loading conditions; in gates, changing the operating state changes the added mass and damping as well | A regime-based, dimensionless description of fluctuating loads on sill-equipped gates, and an efficiency–load criterion, appear not to be available |
| Core question | How can structural poles be separated from operating-state pole shifts and flow-excitation peaks? | How do sill geometry and submergence change load magnitude, frequency and mechanism, and where is the efficiency–load optimum? |
| Working hypothesis | Modelling poles as smooth functions of a measured hydraulic state θ gives unbiased identification across operating states | A sill shifts separation control from the gate lip to the sill crest; frequencies follow regime-specific Strouhal scaling; a favourable trade-off region exists |
| Expected contribution | Parameter-varying transmissibility identification for water-interacting structures | Regime map, scaling relationship and efficiency–dynamic-load criterion |

# 4.  Scope separation

| Component | Zina | Jamal | Shared |
| --- | --- | --- | --- |
| Research gap | Identification of time-varying water-interacting structures | Mechanism and scaling of fluctuating loads | — |
| Scientific question | Separating structural poles from operating-state effects | Load regimes, scaling and the trade-off | — |
| Hypothesis | Parameter-varying pole model λr(θ) | Separation-control shift; regime Strouhal scaling | — |
| Test specimen | Flexible instrumented gate | Rigid gate with interchangeable sills | Gate family / outer geometry |
| Sensors | Accelerometers; impact hammer | Pressure transducers | DAQ hardware |
| Measurements | Accelerations; EMA FRFs | Wall pressures; discharge; depths | Hydraulic-state labels |
| Numerical modelling | Structural FE with added mass; synthetic loads | LES/DES with VOF (CFD) | — |
| Analysis | System identification algorithms | Spectral load analysis; dimensional analysis | — |
| Validation | FE exact poles; EMA ground truth; public dam records | Measured spectra; held-out configurations | — |
| Expected results | Identification method and protocol | Regime map, scaling law, criterion | — |
| Journal papers | Z1–Z3 (own data, first author) | J1–J3 (own data, first author) | Optional joint paper only |

## Independent executability

| If Zina's work is delayed by a year, can Jamal still finish? | Yes. His experiments, CFD and load model use his own rigid specimen and pressure data, and need no modal information. |
| --- | --- |
| If Jamal's work is delayed by a year, can Zina still finish? | Yes. Year 1 is theory and FE; her laboratory runs need only flume flow and her own specimen; Paper Z3 uses public dam records; her loads are synthetic. |

Each thesis is designed to remain independently executable, with clear scope separation and independent data ownership.

# 5.  Shared resources

- Flume and water supply, with shared scheduling. Runs may be simultaneous, but each candidate records their own quantities.

- General gate family and initial drawings, with the same outer dimensions but separate specimens: flexible for Zina, rigid with sill inserts for Jamal.

- DAQ infrastructure and general laboratory setup, with separate channels and files.

- Common baseline hydraulic conditions (head, opening, submergence, discharge), logged once and used by both only as labels.

No measured response, load, model or result is shared in a way that would create dependency. One optional joint paper (for example, a resonance-risk discussion combining load spectra with identified frequencies) could follow later, but it would not form part of either thesis's core contribution.

# 6.  Minimum laboratory requirements

| Item | Zina | Jamal |
| --- | --- | --- |
| Flume | ESSENTIAL: hold still water at 4–6 levels; run 4–6 flow states | ESSENTIAL: tailgate for free/submerged control; adequate discharge range |
| Gate model | ESSENTIAL: one flexible plate gate (light, measurable modes) | ESSENTIAL: one rigid gate; sill inserts (3 heights × 2 lengths) |
| Response / load sensors | ESSENTIAL: 6–8 accelerometers; impact hammer with force sensor. USEFUL: strain gauges. OPTIONAL: laser vibrometer | ESSENTIAL: 6–8 flush pressure transducers with adequate bandwidth. USEFUL: two-component load cell |
| Hydraulic measurements | USEFUL: head and discharge as θ labels | ESSENTIAL: discharge meter; water-level gauges. USEFUL: high-speed camera. OPTIONAL: ADV/PIV |
| DAQ | ESSENTIAL: synchronised, ≥8 channels | ESSENTIAL: synchronised, ≥8 channels (shared hardware possible) |
| Operating-state variation | ESSENTIAL: water level and opening at 4–6 states | ESSENTIAL: opening (3) × flow state (2–3) |
| Minimum configurations | About 4–6 still-water EMA levels + 4–6 flow states (about 20–30 runs) | 7 geometries × 3 openings × 2–3 flow states (about 40–60 runs) |
| Computation | Workstation (FE, identification) | Workstation or cluster for 6–8 LES/DES cases |

The experimental scope is deliberately bounded. Instrumentation requirements will be refined after confirmation of existing laboratory resources.

# 7.  Publication architecture

|  | Zina | Jamal |
| --- | --- | --- |
| Paper 1 | Parameter-varying transmissibility formulation and numerical benchmark | Experimental fluctuating loads and regime identification |
| Paper 2 | Laboratory validation with EMA ground truth | Separation-control mechanism and Strouhal scaling (experiment + LES) |
| Paper 3 | Dam application with reservoir-level-dependent records | Efficiency–dynamic-load criterion; prototype-scale interpretation |

# 8.  Open points and alternatives

- Recent-literature confirmation of both gaps through a short structured review. If the plain-sluice framing of Jamal's concept proves already covered, the same question would be studied for labyrinth sliding gates with sills [5].

- Scope: confirmation that neither topic overlaps with current students' work (for example, in transmissibility identification or gate hydraulics).

- Facilities: flume, pressure transducers, accelerometers and DAQ, available or accessible.

Alternative if laboratory access is not available: both concepts would become identification-based and require no pressure campaign. Zina would study uncertainty-quantified transmissibility identification from seismic dam records; Jamal would study the gate-identification concept, validated numerically and on an accessible regulator gate. If only Zina's topic overlaps with a current student's work, Zina would move to the seismic-uncertainty concept and Jamal's concept would remain unchanged.

## Key supervisor publications referred to

[1] Damadipour M., Tarinejad R., Aminfar M.H. (2021). Seismic modal identification using a new approach based on weighted transmissibility. Earthquake Engineering & Structural Dynamics, 50(7). https://doi.org/10.1002/eqe.3435

[2] Damadipour M., Tarinejad R., Aminfar M.H. (2022). Structural system identification based on combining weighted transmissibility and wavelet transform. Structural Control and Health Monitoring, 29(2), e2868. https://doi.org/10.1002/stc.2868

[3] Tarinejad R., Hafezi A., Pourgholi M. (2025). Automated modal analysis of an arch dam using a hybrid SSI framework based on error analysis and fuzzy clustering. Journal of Vibration and Control.

[4] Tarinejad R., Amanzad F. (2025). Identification of operational modal parameters using multi-reference weighted Hermitian transmissibility functions. Journal of Sound and Vibration, 615, 119212. https://doi.org/10.1016/j.jsv.2025.119212

[5] Abbaszadeh H., Tarinejad R. (2025). Soft computing and predicting labyrinth gates discharge coefficient. Water Resources, 52(4), 701–714. https://doi.org/10.1134/S0097807824604734

[6] Abbaszadeh H., Tarinejad R., Roushangar K., Daneshfaraz R. (2026). Investigation of the effects of internal structure and material properties on the hydraulic performance of rectangular gabion weirs: experimental and numerical study. Water Resources Management, 40, 376. https://doi.org/10.1007/s11269-026-04667-3

[7] Vaseti M.M., Akhtari A.A., Tarinejad R. (2026). Optimized mesh zoning for dam bottom outlet CFD simulations using Richardson extrapolation and grid convergence analysis. Engineering Reports, 8(6). https://doi.org/10.1002/eng2.70800