PRELIMINARY DOCTORAL CONCEPT · PROPOSED RESEARCH DIRECTION

Unsteady Hydrodynamic Loading of Sluice Gates with Bottom Sills

بارگذاری هیدرودینامیکی ناپایدار دریچه‌های کشویی دارای آستانه

Candidate: Jamal · Proposed supervisor: Prof. Reza Tarinejad · Faculty of Civil Engineering, University of Tabriz · October 2026

Status of this concept Preliminary concept for discussion, not an approved topic. Unresolved: (1) a short recent-literature confirmation review of the gap; (2) confirmation that no current student already owns this scope. One half of a proposed paired programme; Zina's concept is independent of it.

## Title options

| A. Discussion | Unsteady Hydrodynamic Loading of Sluice Gates with Bottom Sills |
| --- | --- |
| B. Formal PhD | Hydrodynamic Load Regimes of Sluice Gates with Bottom Sills: Mechanisms, Scaling and the Discharge-Efficiency versus Dynamic-Load Trade-off |
| C. Publication | Regime-based scaling of fluctuating hydrodynamic loads on sill-equipped sluice gates |

# 01  PROBLEM

Adding a bottom sill under a gate is a known way to increase discharge capacity. In the supervisor's group, sills on labyrinth sliding gates increased the discharge coefficient by about 9.9–20.8% depending on sill width (T17). The group's related work (T17, T18, T19) characterises time-averaged performance. A sill also changes where the flow separates (gate lip, sill crest, or both), and therefore the fluctuating loads that drive gate vibration and fatigue. These are unquantified for sill-equipped gates, so an efficiency gain could carry an unmeasured dynamic cost.

# 02  RESEARCH GAP

The general physics of gate flow is not the gap. Vortices, shear-layer oscillation below submerged gates and the effect of sills on mean pressure have all been studied, as the targeted check made during this study showed. The gap is narrower:

- There is no regime-based description of which hydraulic conditions produce strong fluctuating loads on sill-equipped gates.

- There is no dimensionless relationship linking sill geometry, opening and submergence to load magnitude and frequency content.

- There is no criterion identifying operating regions that keep the discharge gain without raising dynamic excitation.

- Fallback if the plain-sluice version is already covered: the same question for the group's labyrinth sliding gates with sills (T17), for which no dynamic-load data were found.

# 03  THE SCIENTIFIC CHAIN (WHY THIS IS NOT A MEASUREMENT CAMPAIGN)

PHYSICAL MECHANISM      which separation / shear-layer process dominates?

│

LOAD CHARACTERISATION   rms magnitude, frequency content, narrow vs broadband

│

DIMENSIONLESS RELATION  which groups and length scale collapse the data?

│

ENGINEERING CRITERION   regions of high efficiency and low dynamic load

Measurements and simulations are means to discover the mechanism and the scaling. The deliverable is the relationship and criterion, tested on held-out configurations.

# 04  HYDRAULIC REGIMES TO BE ESTABLISHED

| Candidate regime (working hypothesis) | Expected dominant mechanism | What must be discovered |
| --- | --- | --- |
| R1 Free flow, no or low sill | Separation at the gate lip; shear layer of the contracted jet | Baseline fluctuating-load level and spectral shape |
| R2 Free flow, effective sill | A second separation point at the sill crest; possible reattachment on the sill depending on its length | The sill height and length at which control of separation shifts from lip to sill |
| R3 Submerged flow | Recirculating roller over the jet; shear-layer feedback that may produce narrow-band oscillation | Whether submergence turns broadband loading into narrow-band loading, and at what submergence |
| R4 Transition free ↔ submerged | Unstable switching between R1/R2 and R3 | Regime boundaries and whether the transition produces load peaks |

# 05  DIMENSIONLESS FRAMEWORK (CANDIDATES, NOT FINAL)

| Group | Role |
| --- | --- |
| a/H1 (relative opening) | Basic control variable |
| s/a, Ls/a, xs/a | Relative sill height, length and position. Geometry controlling separation |
| Submergence ratio (tailwater / opening) | Free vs submerged regime |
| Fr (at the contracted section) | Inertial scaling of the jet |
| St = f·ℓ/V | Frequency scaling. The candidate length scale ℓ is to be determined: contracted jet thickness, lip–sill gap, or sill length |
| C′p, C′F (rms pressure and force coefficients) | Load magnitude normalised by jet dynamic pressure |
| Spectral concentration index | Share of load energy in a narrow band (more critical for resonance) |
| Cd; Re | Hydraulic efficiency, paired with C′F for the trade-off; Re monitored to bound scale effects |

# 06  HYPOTHESES (TO BE TESTED)

- Mechanism. Above a threshold relative sill height, control of separation moves from the gate lip to the sill crest. That shift changes both the level and the spectral shape of the fluctuating load.

- Scaling. Within each regime, the dominant load frequency collapses on a Strouhal number with a regime-specific length scale. Identifying that length scale is a primary outcome.

- Trade-off. There is a range of sill geometry and operating conditions in which most of the discharge gain is kept while C′F and the spectral concentration stay at or below the no-sill reference.

# 07  PREDICTIVE MODELLING DIRECTION

No final equation is proposed in advance. The thesis must discover:

- the regime boundaries in the (s/a, submergence, a/H1) space;

- the length scale that collapses the dominant frequencies in each regime;

- the functional dependence of C′F and of spectral concentration on the sill groups, within each regime;

- whether one efficiency–load criterion holds across configurations, tested on held-out geometries.

The model form will be semi-empirical and regime-conditioned, with the structure set by the mechanism and coefficients fitted to data. Black-box machine learning is not the contribution. CFD explains mechanisms; it does not replace experiments.

# 08  METHODOLOGY AND BOUNDED SCOPE

| Work package | Content | Output |
| --- | --- | --- |
| WP1 Framework | Dimensional analysis, regime hypotheses, reduced test matrix | Test plan |
| WP2 Experiments | One rigid gate with 6–8 flush pressure transducers on the lip, skin plate and sill. Configurations: no sill plus 3 sill heights × 2 lengths (7 geometries) × 3 openings × 2–3 flow states (free, submerged), about 40–60 runs, a subset repeated | Load data; Paper J1 |
| WP3 CFD | LES or DES with VOF at about 6–8 selected cases. Mesh verified with the Richardson/GCI procedure used in T19. Validated against measured spectra | Mechanism evidence; Paper J2 |
| WP4 Model and criterion | Regime map, scaling relationship, efficiency–load criterion, checked on held-out configurations. Prototype-scale interpretation with a scale-effect discussion | Criterion; Paper J3 |

# 09  VALIDATION

- Measured pressure spectra are the primary reference; CFD is accepted only where it reproduces them. The no-sill baseline is checked against classical underflow-gate data (compiled in the confirmation review).

- Predictive check: the relationship and criterion are derived on part of the matrix and tested on held-out configurations.

# 10  EXPECTED CONTRIBUTION AND PAPERS

If the hypotheses hold: a regime-based, dimensionless description of fluctuating loads on sill-equipped gates and a criterion linking the group's discharge-efficiency results to dynamic safety (novelty subject to the confirmation review).

| Paper | Indicative content | Candidate journals |
| --- | --- | --- |
| J1 | Experimental fluctuating pressures and forces; regime identification | J. Hydraulic Research; J. Hydraulic Eng. |
| J2 | Mechanism of separation control and Strouhal scaling (experiment + LES) | J. Fluids Struct.; Physics of Fluids |
| J3 | Efficiency–dynamic-load criterion with prototype-scale interpretation | Water Resources Management; J. Irrig. Drain. Eng. |

# 11  INDEPENDENCE

Rigid gate and pressure sensors only; no modal identification; nothing needed from Zina. Laboratory requirements: programme summary §6.

## Verified references (supervisor's publications)

[1] Abbaszadeh H., Tarinejad R. (2025). Soft computing and predicting labyrinth gates discharge coefficient. Water Resources 52(4), 701–714. doi:10.1134/S0097807824604734 [T17]

[2] Abbaszadeh H., Tarinejad R., Roushangar K., Daneshfaraz R. (2026). Investigation of the effects of internal structure and material properties on the hydraulic performance of rectangular gabion weirs: experimental and numerical study. Water Resources Management 40, art. 376. doi:10.1007/s11269-026-04667-3 [T18]

[3] Vaseti M.M., Akhtari A.A., Tarinejad R. (2026). Optimized mesh zoning for dam bottom outlet CFD simulations using Richardson extrapolation and grid convergence analysis. Engineering Reports 8(6). doi:10.1002/eng2.70800 [T19]